April 29, 1999

METHODOLGY REPORT

TABLE OF CONTENT

1. THE COASTAL CHARS	2

1.1 Their dynamism	2

1.2 Their consolidation	3

2. PHYSICAL ENVIRONMENT	3

2.1 Rainfall	3

2.2 Soils and groundwater	4

2.3 The tide	5

2.4 Surface water levels and salinity	5

3. EVALUATION OF INFRASTRUCTURE	5

3.1 Sea facing embankments	5

3.2 Interior embankments	6

3.3 Sluices	7

3.4 Drainage systems	8

3.5 Fish ponds and fisheries	8

4. DESIGN AND IMPLEMENTATION OF INFRASTRUCTURE	9

4.1 Mapping	9

4.2 Embankments	10

4.3 Sluices and drains	10

4.4 Implementation of works	10

5. WATER MANAGEMENT AND INSTITUTIONAL MATTERS	11

6. OPERATION AND MAINTENANCE	12

METHODOLGY REPORT

# the coastal chars

## Their dynamism

Chars are coastal flats stretching out in front of large sections of the Bangladesh coast. There is a great variety in chars. The youngest chars are mud flats supporting little vegetation, dissected by tidal creeks, subject to frequent flooding during high tides. The oldest chars are already consolidated lands, supporting annual cropping and even more or less permanent homestead, despite the fact that the lands are unprotected and vulnerable to cyclones and so called tidal bores, minor but more frequent cyclones doing extensive damage to the crops.

The great variety in coastal chars already reflect in the diversity among the three CDSP areas, which presently constitute the Project area. The Baggar Donnas are older chars, consolidated and located rather much inland. Their main outlet for drainage is a tidal river which, however, suffers from siltation, char forming, at the mouth. The consequence is that the tide has weakened and does not provide as much drainage opportunities as one would wish. In addition the river carries runoff from the inland and the high discharges during the monsoon suppress the tidal effect. Around the Baggar Donnas the surface water is sweet during extended parts of the year, as long as the discharge from the inland is substantial. Once the discharges decrease the surroundings turn saline. Despite the fact that the Baggar Donnas are older chars, they are not particularly high topographically. Parts have remained relatively low for a number of reasons, which are not known in detail.

Char Bhatirtek, another CDSP area, is rather different. It is a consolidated char, but, again for reasons unknown, it is topographically higher than the Baggar Donnas. It is located along the wide mouth of a tidal river and there is a strong tide along its outer embankments. This provides a good opportunity for drainage. When CDSP started in 1994 and as long as the area was unprotected, fishing was an important activity. The daily flow in its tidal creeks provided great opportunity for fishing. The area was dotted with large fish nets, which have decreased in numbers since the protection was completed.

Char Majid, the third CDSP area resembles Bhatirtek in part and partly it is like the Baggar Donnas. It is not draining into a tidal river but onto mud flats, which are consolidating quickly. The mud flats are still prone to occasional flooding and consequently to siltation. The outfall channel of Char Majid, rather long due to progressing siltation, silts up as well. This renders frequent cleaning a necessity. Frequent cleaning is a maintenance cost. If resources do not come off regularly, the drainage of Char Majid will become problematic.

This is an essential characteristic of the coastal chars: every char to be protected is different. In addition their environment is dynamic, changing all the time. Areas with strong tide may have lost it after years, low areas become high. Other areas, if they are not sufficiently protected, may disappear.

## Their consolidation

The last paragraph of the previous section raises the question, whether chars can be turned into lands that are safe and consolidated enough to support permanent production and permanent living grounds for the population. Part of the answer should be clear: projects like CDSP can not provide secure conditions on their own. Their areas are too small and will always remain susceptible to unexpected and large changes. In addition, the physical environment is only part of CDSP’s concerns. The Project addresses institutional and social development, which is as much needed as physical consolidation.

CDSP kind of projects, need support of large scale physical measures, primarily at two fronts: the sea facing front and its back land. There is great need to know more about the dynamism which surrounds the CDSP areas. Additional large scale studies, like the Meghna Estuary Study, will be needed in the foreseeable future.

There are other developments as well. Since the major cyclones and floods of the late 1980s, which generated the Flood Action Plan, an impressive coastal protection has taken shape. Unconsolidated chars are put under forests on a large scale. They slow down damaging tidal events and are productive in wood. These days, the horizon of the char land is lined with distant forests, which is a major achievement. As far as CDSP is concerned, it means that the Project, sooner or later, will become involved with clearing, reclaiming and planting of coastal forests as part of its activities.

The back lands are more neglected. There is a chance that what is gained on the sea front is lost in the inland. The coast line, moving south, lengthens drainage paths. The siltation blocks river mouths, which can not be cleaned everywhere and all the time. It is important to know, therefore, which rivers are considered sustainable and to those rivers the back lands should be drained. Also that part of char development is beyond CDSP’s control.

# physical environment

## Rainfall

The CDSP polders have changed the physical environment of the areas within their boundaries fundamentally. The major changes occur during the dry season. Before looking into the factors subject to change, unaffected factors will be considered first.

Rainfall has been measured in all three CDSP areas and should be measured in the future in new areas. The rainfall records provide dimension to the Project history. For instance, the depth of a heavy rain, which causes deep flooding in the polders, should approximately be known.

Each record of a CDSP rainfall station is by definition a short term record. That means that, although rainfall amounts are known, there is no long term interpretation. Short term records can not answer the question whether a month was dry, wet or average. The interpretation of short term records    should come from a backup station with a sufficiently long record. For the present Project area the Noakhali station is used. When CDSP will move to the Muhuri area in the future, it will be necessary to find a backup station in that region.

Rainfall exceeds evapotranspiration by far, even during dry monsoons. A low rainfall intensity is about 15 mm/day for periods of 10 days and more. A high rainfall intensity, suitable for drainage design, is about 30 mm/day, which results in a drainage requirement of about 3 l/sec.ha.

## Soils and groundwater

There are no known daily groundwater records in the chars from before CDSP, except those of LRP. The data collected, show that the groundwater falls deep below the surface during the dry season. Another finding is that in all areas a so called deep drainage flow exists, from the char areas towards the sea. The magnitude of this    deep drainage flow can not be known from the data, however.

The monsoon rains and the drainage flow have a limited impact on the salinity of the soils. The soils of the chars contain predominantly fine pores, which retain water strongly by capillary forces, even when the ground water drops low. Originally, the water in the pores is saline. The rains tend to wash the salts out of the soil, but only to a limited depth. Below that depth the water in the pores remains salt, while a decline in the groundwater tables only empties a small portion of the pores. After the monsoon, capillary rise brings the salt back to the upper layers, where it affects the crops.

Empolderment makes the annual salt balance negative. Every monsoon the rains wash the salts from the upper soil layer. They are replaced again by salt from below, during the dry season, but every year the salt has to come from greater depth. In the end, the amount of salts moving up will become negligible. The time required to reach that situation is estimated at about 20 years, but substantial data to confirm that figure are lacking.

In unprotected areas, regular flooding with salt water from the sea, replenishes the salts that wash out during the monsoon. Only after the char has grown sufficiently high above flood level, the salt balance of the upper layer becomes negative and the saline soils may gradually become non-saline. That process, however, takes longer than 20 years.

CDSP’s experience is that the Baggar Donnas already have a higher production potential than Bhatirtek and Majid, which have been protected only for the last few years. That may be due to salts depletion, but the protection provided by the embankments also encourages land users to care better of their soils. So besides salts, farming measures may be important.

A major limitation of soil and water conditions in the CDSP polders is, that boro cropping with irrigation water pumped from the subsoil, is not possible. The deeper groundwater, even that within the reach of a shallow tube well, will remain saline for some time. How long it takes for the groundwater to become sufficiently sweet to support irrigation of a boro crop is also unknown. That may also take longer than 20 years.

## The tide

The tide is important for the CDSP polders. In this respect there is a great difference between the Baggar Donnas on one hand and Bhatirtek and Majid on the other. The tide provides an opportunity to drain the land. Bhatirtek and Majid have sufficient tide for drainage. The Baggar Donnas have a far weaker tide. This is caused by their location inland and by the fact that the tidal river they drain on, has started silting up. In this respect the Baggar Donnas have become part of the old land. During the monsoon, surface water levels inside the polder are equal to those outside.

## Surface water levels and salinity

Surface water levels and the salinity of the surface water are greatly affected by empolderment and this, before all, during the major part of the dry season. After the monsoon, the land users drain their excess water out of the polders up to a certain level. Once that level has been reached, the sluices are closed. The water levels inside the polders then are determined by the evaporation and the deep drainage flow, with a possible component of seepage of outside water through the embankments into the polders, if outside levels remain high for a prolonged period of time. After the monsoon the surface water is sweet. It remains sweet all through the dry season i. e. it retains a quality, suitable for irrigation, not for human or animal consumption. The amount of water contained in the drainage system, however, is too limited to be used for irrigation on a large scale. Land users having land along a drain, use the water for irrigating their rabi crop.

Water levels outside the polders are determined by the sea level and remaining runoff during the dry season. That level may be rather different from the water levels inside the polders. In addition the outside water is saline. So empolderment affects environmental conditions of the surface water inside the polders substantially.

# evaluation of infrastructure

## Sea facing embankments

Most coastal chars are subject to two kinds of flooding: floods from the sea bringing in saline water and floods from the inland bringing fresh water. CDSP polders, therefore, in broad terms, know two kinds of embankments: sea facing and interior embankments.

Flooding with sea water is harmful to the soil. After such a flood, it may take several years before the situation from before the flood is restored. That is the great damage done by tidal bores. They may cause only minor damage to homesteads, with no loss of life. But they damage the crop not only during the flood, but during the following years as well.

Protection against flooding from sea water is essential for advancement of production in the chars. The sea facing embankments provide that protection. They also provide protection against major damaging cyclones. Present developments, with sea facing embankments, together with the extensive coastal forest belts, provide a great degree of safety.

## Interior embankments

The interior embankments of the CDSP polders provide protection against flooding from the inland. The water of the floods is sweet and may cause instantaneous damage, but no damage on longer term to the soil. The Baggar Donnas were fully protected against runoff, except for some culverts in the eastern embankments, which still allow outside water in. These did not cause major problems, due to their limited size.

However, particularly Baggar Donna II is suffering from another development. People living outside the polder, watching the better protection provided in the CDSP area, have cut the interior embankment and drain their excess water via the polder. Such public cuts threaten the protection against runoff, not so much because they can not be controlled physically, but because the outside population does not allow them to be closed.

A complication is, that it is not fully clear to what extent the public cuts alleviate the flooding situation outside or increase the flooding inside. The damage done to the crops during the 1998 floods must be largely blamed on the    weak tide in the area. Baggar Donna I and II were more or less equally affected, while Baggar Donna II was more exposed to the flood water from the north.

Although no public cuts have occurred in Char Majid, the polder is also vulnerable to it. However, a provision has been made from the beginning. Existing culverts in the northern embankment have been maintained and water from the outside still can enter. The culverts have not been enlarged but largely left untouched. In addition, in the capacity of the Majid sluice, drainage water from outside areas has been accounted for. A consequence was that within the polder, the area along Bashkali khal, which receives most of the outside water, had damage during the 1998 flood. The damage was not as extensive as in the Baggar Donnas, despite the fact that the sluice was not yet fully functional. The capacity of part of the Bashkali khal is probably to be blamed.

The Nabagram and Kalmi areas in Bhatirtek did not suffer from flooding caused by runoff from the outside areas during 1998. The areas are rather high, have a good tide and the drainage system and the sluices could cope with the quantities coming in.

In summary, CDSP areas should be prepared to receive a limited amount of runoff from the outside. That is realistic. Watersheds immediately upstream of the polders are usually small. The larger watersheds drain via rivers which pass by the polders. In addition, most outside watersheds originally had their drainage via the lands of the    polder, before it existed. An essential element is, that the polders have a good tide which provide drainage opportunities. With a weak tide, as in the Baggar Donnas, drainage is always problematic, whether outside water enters or not. The design of the drainage systems and the sluice should take drainage of the outside areas into account.

The drainage of the areas surrounding the CDSP polders, is part of the larger drainage issue of the interior. The more the coast line moves away from the polders, the more the tide will weaken and the more the polders will become part of the old land. In the meantime CDSP should try to make compromises with its surroundings to have as little damage inflicted as possible. That process is ongoing in the Baggar Donnas and violent conflicts have been avoided. Deliberations with Local Government proved most helpful.

The compromises in Bhatirtek and Majid were to keep boundaries open and account for the incoming water in the design. Nevertheless, all parties should start realising that gradually the interior embankments become redundant. This may also be true for Majid and Bhatirtek. Interior embankments become elevated roads.

## Sluices

The sluices provide protection as part of the embankments, but before all, they provide facilities to manipulate water levels inside the polders. Not only can water be drained, but it can be retained as well, when required.    This facility can be fully used when    there is sufficient tide and the sluice can discharge freely. But also in cases in which the tide is weak, the sluices prove their value.

Due to the weak tide, manipulation of water levels is not well possible during the monsoon in the Baggar Donnas. However, immediately before and after the monsoon, when outside water levels fall. The water on the inside can be manipulated. During the heart of the dry season, the gates remain closed. The outside water is considerably higher than the water inside and it is saline. The water inside remains rather sweet.

Because of the tide in Bhatirtek and Majid, water levels can be manipulated there as well, during the monsoon. Outside the monsoon, the use of the sluices is much the same as in the Baggar Donnas. In summary, operation of the sluices has three major benefits, which should be fully appreciated:

- Water can be retained such that fields do not loose their water too quickly. There is a limit to this. Once water levels have fallen too low, they can not be restored unless there is rain

- During the dry season, the water inside the polders remains far sweeter than that outside. Water quality is suitable for irrigation but marginal for human and animal consumption.

- One may expect that better control of water levels inside the polders, together with sweeter water, stimulate the desalinisation process of the soil.

In this context three comments need to be made. First, Water Management Committees quickly perceive the benefits of sluice operation. They do not have to be trained in that respect. Second, in leaving sluice operation to the WMCs, they choose for agriculture and not for capture fisheries. This comment will be taken up again later.

The third comment touches a rather basic issue. The Water Board traditionally designs sluices with flap gates only. The overriding philosophy is that saline water should be kept out, while there is no concern about retention of sweet water on the inside. Flap gates drain an area in an uncontrolled way. The lowest tide determines the water level inside. That is often too low. With low water tables, rice fields loose their water too quickly and land users are inclined to block drains. With sliding gates this becomes different. Land users could agree on a water level to be maintained.

Sluices in the CDSP areas of the future, which presumably include areas, where sluices have been rehabilitated under CERP, may have flap gates only. CDSP needs to build up a strong position to propagate the introduction of the manually controlled sliding gates. It may take quite some effort to convince the Water Board. Discussions between Water Board staff and land users in the Nabagram and Kalmi areas, to be initiated by CDSP, may be most helpful.

## Drainage systems

Tidal flats have tidal creeks by which the water enters and leaves. When tidal flats are turned into polders, the most practical thing to do is to use the creeks as drains. This saves excavation but it also saves quarrelling about such issues as property rights and land acquisition, whether claims are legal or not. In most unprotected consolidated chars, where cropping has already started, the tidal creek system is still in place, as it is essential for monsoon drainage.

Tidal creeks are formed by nature. Drainage engineers use design criteria such as, that defined units of land, for instance 100 hectare or less, should have access to the drainage system. This usually leads to higher drain densities than the tidal creeks provide. The lesson learned during CDSP is, that the density of the tidal creeks, provides adequate drainage under the circumstances. For the time being at least, there is no urgent need to construct dense new systems.

LRP provided a    rather dense drainage system in Baggar Donna I. This was related to the issue of settling co-operative societies on pieces of land of about 30 ha. Each piece of land had access to the drainage system, which resulted in a rather dense system.

According to LRP’s own conclusions, the dense system did not lead to a quicker desalinisation of the soils. In addition to that, CDSP tends to conclude that the dense drainage system does not lead to appreciably higher yields, although the matter should have been monitored more extensively.

Why this is so is not fully clear. A possible explanation is that land users in Bangladesh have different cropping patterns for high, low and medium land. The high lands drain towards the low lands and the cropping there is adapted, by a different timing and/or a different variety or crop. Drainage designers reason from a different point. Their assumption is that all lands, high and low, should be suitable for optimal cropping. The lower lands then are often the norm. If, under such conditions, the high lands drain too quickly, the land user should take refuge to on-farm measures, better levelling, better bunds, etc.

The paragraphs above should not be interpreted as saying that the present situation is optimal. It intends to say that under the current farming practices, land users generally do not ask for a more intense drainage system. What land users ask for is manual control of the sluice in the Kalmi area. That was their priority.

## Fish ponds and fisheries

Although fish ponds belong only partly to the infrastructure, borrow pits may be considered infrastructure, they can best be discussed here. In unprotected land, subject to frequent flooding, fish ponds are unattractive. The floods provide an opportunity for the fish to escape. In addition, on unprotected land the potential of capture fisheries may be still substantial.

Empolderment changes all that. The embankments are a serious obstruction to fish migration into the polders. The sluices, the only entrance for fish, are closed part of the time. In addition, as has been discussed, empolderment changes the fish environment in the polders to a large extent. Water tables may be different during a most of the dry season and the water remains sweet. Judging from the reactions of the land users in the polders, their priority is not fishing but cropping.

CDSP should acknowledge that capture fisheries decrease in protected areas. To what extent that is the case is unknown, due to the difficulty of assessing the amounts of fish caught, before and after empolderment.

Protection, however, provides good opportunities for fish cultivation in ponds. The ponds will not be flooded any more and water quality can be better controlled. The great limitation is that ponds fall dry, during the dry season. However, the time at which they fall dry, varies greatly among ponds. It can be delayed by excavation. The groundwater data suggest that excavation to about 3 m is a sensible proposition. Deeper ponds may develop problems with the stability of their slopes.

Loss of water from the ponds is generally blamed on seepage. That may be slowed down by providing organic food to the fish, which would better seal the bottom and slopes. However, evaporation is by far a larger actor with respect to the loss of water. In addition, there may be ponds which have positive seepage, certainly when they are located near to embankments and the outside water remains high. Details about the factors which determine the loss of water from fish ponds are largely unknown and may be investigated. However, the targets of a such a campaign need to be better defined.

In the meantime fish ponds do not solve the problems of people who spent part of their time on capture fisheries before empolderment, or, who depended on catching fish for their diet. Those issue should be brought up more clearly in CDSP, than has been done so far.

# design and implementation of infrastructure

## Mapping

From its early beginning CDSP has used existing maps, which were generally of low quality. An obvious difficulty is the dynamism of the area. Maps become obsolete rather quickly. Deficiencies in mapping have plagued the Project up to the present day. In the meantime, facilities for improved mapping have improved in the country during the past years.

Priority is to obtain accurate maps of road and khal systems in CDSP areas and surrounding areas, which are of importance. For instance, the total area draining through the Majid sluice should be better assessed. Roads are important as boundaries for hydrological or WMC areas. If CDSP becomes actively involved with certain areas, an accurate topographical map with elevations is indispensable.

## Embankments

The quality of the design and implementation of infrastructure is not subject of this report. It should be pointed out, however, that in cases, CDSP’s approach leads to somewhat strange solutions. The point in case is Char Majid. Due to its exposed position, on the outside of the existing sea facing embankment, a great length of an additional    sea facing embankment had to be built for a relatively small area.

The situation in the Baggar Donnas is different because CDSP was faced with an existing situation. But also there, certainly considering what has been said about the usefulness of interior embankments, namely that they tend to become redundant, a relatively great length of such embankments has been built for a small area. Char Bhatirtek, again is different. The embankments constructed there, fit into an overall plan of protection and they serve larger areas than the CDSP area alone. The length of interior embankments in Bhatirtek is small.

A point to be considered in the future, moving to new CDSP areas, is, that the infrastructure of such areas should fit into a larger, possibly already existing, planning. If heavy infrastructure is to be built, such as sea facing embankments, it should serve sufficiently large areas.

## Sluices and drains

There are no particular comments to be made about the technical design of the sluices and drains within the framework of this report. However, there is the observation that the sluices of the Majid and the Nabagram areas probably have a considerable overcapacity. Worse is that CDSP never received the details on which the designs were based. The most striking unknown design criteria are: the sizes of the areas draining through drain sections and sluices and the drainage requirements used. In addition, considerations about the amplitude of the tide at the locations of the sluices are lacking.

With respect to drainage design, drawings became available after the works had been completed. There was no opportunity to discuss matters with the Polder and Sub-Polder committees. The deposition of the spoil, resulting from the excavation was never considered during the design and had to be done in a haphazard way during excavation.

On the whole, the evaluation of design procedures and the quality of the designs is negative. It is recommended to make use of local consultant, working under design contracts, specifying requirements. CDSP and the Water Board should comment on, approve or disapprove of the designs.

## Implementation of works

The implementation of the works exceeded the planning in many cases. That may be due to an unrealistic planning, which may not have been discussed with Water Board at all. It could also be to defaulting contractors, with insufficient supervision to remedy the situation in time.

An important consequence was that protection in Bhatirtek and Majid was achieved much later than envisaged. This had consequences for the measuring programmes, the extension programmes, social development programmes etc. It also had consequences for the land users, delaying developments in their cropping.

As with the design of the works, the recommendation is that the role of local consultants is increased. CDSP does not have the manpower and has different tasks, the performance of the Board is inadequate. This may appear more complex than it sounds. Increased involvement of local consultants may have to be gradual. That is no argument, however, not to do it at all.

# WATER MANAGEMENT AND INSTITUTIONAL MATTERS

CDSP’s institutional framework has deviated, from the beginning, from that of the guidelines of the Ministry of Water Resources. The important reason was that CDSP had multi-sectoral activities, which would not easily fit into a water management framework. In addition the 1994 guidelines were published in the same year CDSP started and one can not expect that publishing the guidelines makes them accepted all over the country and at the same time.

During its next phase, CDSP will still be multi-sectoral and the creation of Polder Committees (PC) and Sub-Polder Committees (SPC) remains essential for planning. However, the Project should form Water Management Committees (WMC) earlier, than it did during the previous phases. As a matter of fact the creation of WMCs should be part of the establishment of PCs and SPCs. That saves time and efforts and creates less confusion. That would mean that the formation of PCs, SPCs and WMCs becomes one activity.

The additional advantage is that water management issues can be discussed with the population from the beginning. During the coming year, CDSP intends to start working in areas with existing infrastructure.    Also that circumstance renders it important that water management issues are taken up quickly. In addition, CDSP’s objectives and methodology for water management are much clearer now than they were during its initial days.

For monitoring purposes, CDSP has created so called Water Management Areas (WMA), areas small enough to be considered as a unit and to be easily overseen by the local groups being interviewed. That approach is successful. So far, CDSP has maintained the requirement that WMAs are hydrological unit and in this respect they differ from the SPCs, which are of about the same size.

One may question in the first place, whether hydrological boundaries in the char situation should be a strong criterion. Therefore, during the future formation and definition of PCs, SPCs, WMCs and WMAs, an attempt should be made to let hydrological boundaries and SPC boundaries coincide wherever possible. That simplifies matters.

# operation and maintenance

For all practical purposes operation of the polder infrastructure is not a time consuming matter. It mainly boils down to operation of the sluices, which remain permanently open or closed during prolonged periods of the year. The WMCs should take care of that themselves and the recent withdrawal of the Water Board from sluice operation should be endorsed. This does not mean that sluice operation is not important. It is crucial, but WMCs are aware of that and are capable of handling the matter.

As far as maintenance is concerned, CDSP at present has a maintenance plan for all its areas. The plan goes into details about the infrastructure to be maintained, the expected costs of maintenance and who does the maintenance. It is important to note that the maintenance plan makes a distinction between formal maintenance responsibilities and cost sharing for maintenance. Parties who do not have formal responsibilities, may share in the cost. To that extent memoranda of understanding have been drafted, agreed upon and signed by participating parties.

Maintenance plans are important and are a start on a long road to better operation and maintenance. Parallel to the plan, campaigns are to be conducted to create a better maintenance awareness among the population.

Contained in the maintenance plan are CDSP’s guidelines for operation and maintenance. The guidelines do not run fully parallel to the latest government guidelines. As long as the government guidelines are not yet final, there is no great urgency to adapt them. On the other hand, CDSP should remain aware of the government guidelines and follow and implement them as much as is practically possible.